1A. SOIL SENSOR:
Const int sensor_pin = A1;/* Soil moisture sensor O/P pin */

void setup()

{ Serial.begin(9600);/* Define baud rate for serial communication */ }

void loop()

{

float moisture_percentage;

int sensor_analog;

sensor_analog = analogRead(sensor_pin);

moisture_percentage = ( 100 - ( (sensor_analog/1023.00) * 100 ) );

Serial.print("Moisture Percentage = ");

Serial.print(moisture_percentage);

Serial.print("%\n\n");

delay(1000);

}

1B. GAS SENSOR:

int sensorValue;

int digitalValue;

void setup()

{

Serial.begin(9600); // sets the serial port to 9600

pinMode(13, OUTPUT);

pinMode( 3, INPUT);

}

void loop()

{

sensorValue = analogRead(0); // read analog input pin 0

digitalValue = digitalRead(2);

if(sensorValue>400)

{

digitalWrite(13, HIGH);

Serial.println(sensorValue, DEC); // prints the value read

Serial.println(digitalValue, DEC);

}

else

digitalWrite(13, LOW);

Serial.println(sensorValue, DEC); // prints the value read

Serial.println(digitalValue, DEC);

delay(1000); // wait 100ms for next reading

}

1C. LIGHT SENSOR:

int LDR_Pin = A0;      // LDR connected to A0

int threshold = 500;   // Set threshold value

void setup()

{

Serial.begin(9600);

}

void loop()

{

int ldrValue = analogRead(LDR_Pin);   // Read LDR value

Serial.print("LDR Value = ");

Serial.print(ldrValue);

if(ldrValue > threshold)

{

Serial.println("  -> LOW LIGHT / DARK");

}

else

{

Serial.println("  -> BRIGHT LIGHT");

}

delay(1000);   // 1 second delay

}

1D. SOUND SENSOR:

int soundPin = 10;

bool alarm = false;

void setup()

{

Serial.begin(9600);

pinMode(soundPin, INPUT);

}

void loop()

{

if(digitalRead(soundPin) == LOW)

{

if(!alarm)

{

Serial.println("LOUD, LOUD");

alarm = true;

}

}

else

{

if(alarm)

{

Serial.println("quiet");

alarm = false;

}

}

delay(100);

}

1E. FLAME SENSOR:

int flameSensorPin = 2;   // Flame sensor connected to pin 2

int flameValue = HIGH;

void setup()

{

Serial.begin(9600);

pinMode(flameSensorPin, INPUT);

}

void loop()

{

flameValue = digitalRead(flameSensorPin);   // Read sensor value

if(flameValue == LOW)   // Flame detected

{

Serial.println("FLAME, FLAME, FLAME");

}

else                    // No flame

{

Serial.println("no flame");

}

delay(500);

}

1F. ULTRASONIC SENSOR:

const int trigPin = 9;

const int echoPin = 10;

long duration;

int distance;

void setup()

{

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

Serial.begin(9600);

}

void loop()

{

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH);   // Time taken by echo

distance = duration * 0.034 / 2;     // Distance in cm

Serial.print("Distance = ");

Serial.print(distance);

Serial.println(" cm");

delay(500);

}

2. RELAY INTERFACING:

int relay_pin = 7; int led_pin = 12;

void setup()

{

pinMode(relay_pin,OUTPUT);

pinMode(led_pin,OUTPUT);

digitalWrite(led_pin,HIGH);

}

void loop()

{

digitalWrite(relay_pin,HIGH);

delay(2000);

digitalWrite(relay_pin,LOW);

delay(2000);

}

3. READ & WRITE IN CLOUD:

(a) LED CONTROL:

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

char ssid[] = " ";

char pass[] = " ";

#define LED_PIN D4 // Built-in LED

BLYNK_WRITE(V0)

{

int value = param.asInt();

if(value == 1)

digitalWrite(LED_PIN, LOW); // LED ON

else

digitalWrite(LED_PIN, HIGH); // LED OFF

}

void setup()

{

Serial.begin(9600);

pinMode(LED_PIN, OUTPUT);

digitalWrite(LED_PIN, HIGH); // LED OFF initially

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

}

void loop()

{

Blynk.run();

}

(b) GAS SENSOR:

#define BLYNK_TEMPLATE_ID "Your_Template_ID"

#define BLYNK_TEMPLATE_NAME "Your_Template_Name"

#define BLYNK_AUTH_TOKEN "Your_Auth_Token"

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

// WiFi credentials

char ssid[] = "Your_WiFi_Name";

char pass[] = "Your_WiFi_Password";

BlynkTimer timer;

// Function to send sensor data to Blynk

void sendSensorData() {

int sensorValue = analogRead(A0); // Read the gas sensor value

Blynk.virtualWrite(V1, sensorValue); // Send value to Virtual Pin V1

// Optional: Print to Serial Monitor for debugging

Serial.print("Gas Level: ");

Serial.println(sensorValue);

}

void setup() {

Serial.begin(115200);

// Initialize Blynk

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

// Setup a function to be called every 1000ms (1 second)

timer.setInterval(1000L, sendSensorData);

}

void loop() {

Blynk.run();

timer.run();

}

4. SERVO MOTOR CONTROL:

Program

#define BLYNK_TEMPLATE_ID "YourTemplateID"

#define BLYNK_DEVICE_NAME "ServoControl"

#define BLYNK_AUTH_TOKEN "YourAuthToken"

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

#include <Servo.h>

char ssid[] = "YourWiFiName";

char pass[] = "YourPassword";

Servo servoMotor;

BLYNK_WRITE(V0)

{

int angle = param.asInt(); // Read slider value

servoMotor.write(angle); // Rotate servo

}

void setup()

{

Serial.begin(9600);

servoMotor.attach(D4); // GPIO pin

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

}

void loop()

{

Blynk.run();

}

5. PWM PULSE:

#define BLYNK_TEMPLATE_ID "TMPL3uk0YtMzo"

#define BLYNK_TEMPLATE_NAME "LED"

#define BLYNK_AUTH_TOKEN "A9zRQLJHe7QDL9YmnCWMWe-nwzgHhevG"

#include <WiFi.h>

#include <BlynkSimpleEsp32.h>

char ssid[] = "Vivo Y56";

char pass[] = "Kanish@123";

#define LED_PIN 17

#define PWM_CHANNEL 0

#define PWM_FREQ 5000

#define PWM_RESOLUTION 8 // 0–255

void setup()

{

Serial.begin(115200);

// Setup PWM

ledcAttach(LED_PIN, PWM_FREQ, PWM_RESOLUTION);

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

}

BLYNK_WRITE(V1)

{

int brightness = param.asInt(); // 0–255

Serial.println(brightness);

ledcWrite(LED_PIN, brightness);

}

void loop()

{	Blynk.run(); 	}